M Series Gate Reader

User Manual V1.2

Table of contents

1. Docking Communication Protocol	3

1 .1 Serial communication parameters	3

1 .2 Data frame format	3

2 Command Frame Set	5

2.1 Command List	5

2.2 Module general control commands	7

2.2.1 Initialize device	7

2.2.2 Device factory reset	7

2.2.3 Set RF output power...................................	8

2.2.4 Set / Read the RF protocol standards supported by the module	9

2.2.5 Set / Read Device Network Port Information	10

2.2.6 Set / read device remote network port information	11

2.2.7 Get module information	13

2.2.8 Set Configurable Operating Parameters 	14

2.2.9 Read configurable operating parameters	20

2.2.10 Set read input and output control	20

2.2.1 1 Set / Read WiFi information 	22

2.2.1 2 Set read permission 	24

2.2.1 3 Release /Close Relay 1	25

2.2.1 4 Release /Close Relay 2	26

2.2.15 Set the RSSI filter threshold for reading each antenna port 	27

2.3 International standard (ISO 18000-6 C ) protocol related commands	28

2.3.1 Tag Inventory	28

2.3.2 Stop Inventory	30

2.3.3 Read tag data	31

2.3.4 Select the tag to be operated	32

2.3.5 Set Select command parameters	33

2.3.6 Get Select command parameters 	35

2.3.8 Get Query command parameters	37

2.3.7 Set Query command parameters 	37

2.4 GPIO Control Commands	39

2.4.1 Set /Get GPIO working parameters	39

2.4.2 Get GPIO input and output levels	42

2. 5 Door access control commands	44

2.5.1 Get GATE Status	44

2.5.2 Set / Get Gate Channel Working Parameters 	46

2.5.3 Set /Get Gate Channel EAS Specific Matching Data	50

Appendix A. Operational Status Returned by Tags	51

Appendix B. CRC16 checksum reference C code	52

Appendix C. Definition of STATUS.	53

1. Docking Communication Protocol

This module can communicate with external applications through serial ports RS232/TTL, RS485 and network port RJ45 .

1 .1 Serial communication parameters

The module communicates with external applications through the serial port

Table A-1 Module connection serial port parameters

| project | parameter | Remark |
| --- | --- | --- |
| Baud rate | 115200bps |  |
| Data bits | 8 |  |
| Stop bits | 1 |  |
| Parity bit | None |  |

1 .2 Data frame format

The data packet sent by the upper layer application is called "command" below, and the data packet returned by the module to the upper layer application is called "response" below. The length unit of all the following data segments is bytes.

Data transmission order: For each data item consisting of multiple bytes, the most significant byte is sent first, and the least significant bit is sent first within the byte.

The data frame format of the command is shown in Table A-2:

Table A-2 Command data frame format

| MSB |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- |
|  | Control Field |  |  | Information Field | EpLENogue Field |
| HEAD | ADDR | CMD | LEN | Data[ ] | CHECK |
| 1 Byte | 1 Byte | 2Bytes | 1 Byte | 0~255Byte | 2 Byte |

The response data frame format is shown in Table A-3:

Table A-3 Response data frame format

| MSB |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- |
|  | Control Field |  |  | Information Field |  | EpLENogue Field |
| HEAD | ADDR | CMD | LENS​ | STATUS | Data[ ] | CHECK |
| 1 Byte | 1 Byte | 2Bytes | 1 Byte | 1 Byte | 0~254Byte | 2 Byte |

The data frame structure consists of the following three parts: frame control field (Control Field), information data field (Information Field) and frame tail field ( EpLENogue Field).

|  |  | length (byte) | illustrate |
| --- | --- | --- | --- |
| Control Field | HEAD​ | 1 | HEAD is fixed as the hexadecimal number 0x CF , which is used for data frame synchronization. |
|  | ADDR​ | 1 | Reader address. Address range: 0x00~0xFE, 0xFF is the broadcast address. The reader only responds to commands with the same address as itself and 0xFF. The address of the reader is 0x00 when it leaves the factory. |
|  | C MD | 2 | Command code and command word definitions are shown in Table A-7 . |
|  | LENS​ | 1 | This length is the length of all bytes of the actual Information Field . The maximum data length is 255 bytes ( 0xFF ). |
| Information Field | S TATUS | 1 | STATUS is the execution status of the host computer command contained in the module's response . STATUS only exists in the response data frame returned by the module. There is no STATUS part in the command frame sent by the upper application . STATUS is 0 , which means that the command is executed successfully. Here, successful execution only means that the module successfully receives the tag response. If there is a tag execution status in the tag response, it is necessary to further determine whether the tag execution status is correct. The definition of STATUS is shown in Table A-6 . |
|  | Data[ ] | indefinite | Parameter field. In actual commands, it may not exist. Data[ ] is the actual data to be transmitted. The valid bytes of Data[ ] will be specifically defined in each command format . |
| EpLENogue Field | MSB -CRC16 | 1 | CRC16 high byte. CRC16 is the CRC16 value from HEAD to Data[] . |
|  | LSB -CRC16 | 1 | CRC16 low byte. |

See Appendix B for the CRC16 reference code .

2 Command Frame Set

The UHF RFID reader/writer module is referred to as "module" in the following description.

2.1 Command List

Table A-7 Control command example list

| Command Name | CMD control byte | Functional Description | Remark |
| --- | --- | --- | --- |
| EPC Directive |  |  |  |
| RFM_INVENTORYISO_CONTINUE | 0x00 01 | Label inventory ( process ) |  |
| RFM_INVENTORYISO_STOP | 0x00 02 | Stop label counting |  |
| RFM_READISO_TAG | 0x0003 | Read tag data |  |
| RFM_SETISO_SELECTMASK | 0x0007 | Set the label EPC number |  |
| RFM_SET_SELPRM | 0x00 10 | Select parameter settings |  |
| RFM_GET_SELPRM | 0x00 11 | Select parameter acquisition |  |
| RFM_SET_QUERY_PARAM | 0x00 12 | Query parameter settings |  |
| RFM_GET_QUERY_PARAM | 0x00 13 | Query parameter acquisition |  |
| Module custom directives |  |  |  |
| RFM_MODULE_INT | 0X0050 | Initialize the device | Stop all actions |
| RFM_REBOOT | 0x00 52 | Restore factory settings | Restore Defaults |
| RFM_SET_PWR | 0x0053 | Setting RF Power | 0-26dBm, all values above 26 are 26 |
| RFM_SET_GET_RFIDPRO | 0x0059 | Set/get RF protocol | Currently the device only supports ISO 18000-6C |
| RFM_SET_GET_NETPARA | 0X005F | Set/get network port working information | Applicable to LAN |
| RFM_SET_GET_REMOTE_NETPARA | 0x0064 | Set/get remote network port working information | Applicable to the entire network, cross-segment remote |
| RFM_GET_DEVICEINFO | 0X0070 | Get all module version numbers and sn codes |  |
| RFM_SET _ALL_PARAM | 0x0071 | Set all basic parameters | Package all basic parameters and set them all at once |
| RFM_ GET_ALL_PARAM | 0x0072 | Read all basic parameters | Package all basic parameters and read them all at once |
| RFM_SET_ GET_IOPUT _PARAM​ | 0x0074 | Set/ get input and output working parameters |  |
| RFM_SET_GET_WiFi_PARAM | 0x0075 | Set/ get WiFi working information |  |
| RFM_SET_GET_S- PERMISSION_PARAM | 0X0076 | Set/ get specific TAG read permission |  |
| RFM_RELEASE_CLOSE_RELAY1 | 0X0077 | Release/Close Relay 1 |  |
| RFM_RELEASE_CLOSE_RELAY2 | 0X0078 | Release/Close Relay 2 |  |
| RFM_SET_GET_AntN -RSSI-Filter | 0x0079 | Set/ get RSSI threshold for each antenna |  |
| GPIO Control Commands |  |  |  |
| RFM_SET_GET_G PIO_WORKPAMAR​​ | 0x0080 | Set/get GPI O working parameters |  |
| RFM_GET_G PIO_LEVEL​​ | 0x0081 | Set /get GPIO level |  |
| Door access control commands |  |  |  |
| RFM_GET_GATE_STATUS​​​​ | 0x0082 | Get the GATE channel gate status |  |
| RFM_SET_GET_GATE_PARAM​​​ | 0x0083 | Set/get GATE channel gate working mode |  |
| RFM_SET_GET_EAS_MASK​ | 0x0084 | Set/get EAS specific match data |  |

2.2 Module general control commands

2.2.1​ RFM_MODULE_INT ( Initialize device )

The device is initialized and configured according to the stored parameters.

- Format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x00 5 0 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x 00 | 0x00 5 0 | 0x01 | 1 Byte | 2Byte |

 STATUS: This command will only return a successful execution (value 0x00), other values are invalid;

2.2.2​ RFM_REBOOT ( device factory reset )

Restore factory settings command. When this command is successfully executed, the module will be restored to factory settings (does it need to be restarted?) .

- Format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x00 52 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x 00 | 0x00 52 | 0x01 | 1 Byte | 2Byte |

 STATUS: This command will only return a successful execution (value 0x00), other values are invalid;

2.2.3​​ RFM_SET_ PWR 	( Set RF output power )

This command is used to set the module's RF output power. Before using the module to operate the tag, the user needs to use this command to set the module's RF output power. If the user does not set the module's RF output power, the module will use the default setting when it works.

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Power | Resv |  |
| 0xCF | 0xFF​ | 0x00 53 | 0x02 | 1 Byte | 1 Byte | 2Byte |

  - Power: output power, in dBm, value range: [0, 26 ]dBm, values greater than 26 dBm are 26 dBm .

  - Resv : System reserved field, default value is 0x00;

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 53 | 0x01 | 1 Byte | 2Byte |

  - STATUS:

  - 0x00: command executed successfully;

  - 0x01: The module does not support the output power value;

  - Other values: invalid;

2.2.4​​ RFM_SET_GET_RFID PRO ( Set / read the RF protocol standards supported by the module )

This command sets/reads the module's radio frequency RFID protocol standard specification ( by issuing this command, the protocol standard can be set to determine whether the module is successfully connected ).

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | R FIDPRO |  |
| 0xCF | 0xFF​ | 0x00 59 | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

  - Option: command control option

0x01: Set , followed by RFIDPRO of 1 Byte in length ;

0x02: Read , then do not connect to RFIDPRO;

Other values: invalid;

  - RFIDPRO : Protocol options , see the table below for details.

- Response format and status byte

Set the response:

| HEAD | ADDR​ | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0059 | 1Byte | 1Byte | 1Byte | 2Byte |

获取响应：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | RFIDPRO |  |
| 0xCF | 0x00 | 0x0059 | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte |

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: Command execution successful ; 0x01 : Parameter error. |
| Option | 1 | 0x01: Set; 0x02: Get |
| RFIDPRO | 1 | 0x00: ISO 18000-6C; 0x01: GB/T 29768 ; 0x02: GJB 7377.1 ; Currently only ISO is supported 18000-6C. |

2.2.5 RFM_ SET_GET_NETPARA ( Set / Read Device Network Port Information )

This command sets/reads the module's network port information. The default setting is TCP Service, the host computer connection needs to set the corresponding device IP address and port;

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | NetPara |  |
| 0xCF | 0xFF​ | 0x00 5F | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

  - Option: command control option

0x01: Setting;

0x02: read;

Other values: invalid;

- Response format and status byte

Set the response:

| HEAD | ADDR​ | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 5F | 1 Byte | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | ADDR​ | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x 00 | 0x000 5F | 1 Byte | 1 Byte | 1 Byte | NByte | 2Byte |

| Fields |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: Command execution successful ; 0x01 : Parameter error. |
| Option |  | 1 | 0x01: Set; 0x02: Get |
| NetPara | IPAddr | 4 | Device IP address. If the IP address is 192.168.1.1, the data is [ 0xC0, A8, 0x01, 0x01] |
|  | MacAddr​ | 6 | Mac address, if multiple devices work at the same site, please use different MAC addresses; For example: mac[6]={0x00,0x08,0xdc,0x11,0x11,0x11} , the mac address is 00-08-DC-11-11-11 |
|  | Port | 2 | Listening port, the value is [ 0,65536] , the default is 9527; |
|  | NetMask | 4 | Subnet mask, default is [0xFF,0xFF,0xFF,0x00] |
|  | GateWay | 4 | Default gateway, default is [0xC0,0xA8,0x01,0x01] |

2.2.6​​ RFM_SET_GET_REMOTE_NETPARA (set / read device remote network port information )

This command sets/reads the remote network port information. The device sends data to the remote IP , and the remote computer can monitor and receive the data.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | NetPara |  |
| 0xCF | 0xFF | 0x00 64 | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

  - Option: command control option

0x01: Setting;

0x02: read;

Other values: invalid;

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x00 5F | 1 Byte | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x00 | 0x000 5F | 1 Byte | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

| Fields |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: Command execution successful; 0x01: Parameter error. |
| Option |  | 1 | 0x01: Set; 0x02: Get |
| NetPara | Enable | 1 | Enable switch, 1: enable sending, 0: disable sending. |
|  | IPAddr | 4 | Remote IP address. If the IP address is 192.168.1.1, the data is [0xC0, 0xA8, 0x01, 0x01] |
|  | Port | 2 | Listening port, the value is [0,65536], the default is 9527; |
|  | HeartTime | 1 | Heartbeat packet time, to prevent disconnection, unit*5S |

2.2.7​ RFM_GET_ DEVICE INFO (Get module information)

This command is used to obtain the version information of the current device , including the hardware version number, firmware version number, and SN number of the CP module and RFID module .

- Format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0x CF | 0xFF​ | 0x00 70 | 0x00 | 2 Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | PAYLOAD |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | CPHardVer | CPFirmVer | CPSN_code |
| 0xCF | 0x00 | 0x0070 | 0xAC | 1Byte | 32Bytes | 32Bytes | 12Bytes |

| PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- |
| RFIDModeVer | RFIDModeName | RFIDMode_SNCode |  |
| 32Bytes | 32Bytes | 1 2 Bytes | 2 Byte |

Definition of each byte:

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | This command will only return a successful result (value 0x00), other values are invalid; |
| CP HardVer | 32 | CP module hardware version number , in ASCII format, ending with ' \0 ' . The actual hardware version number may be less than 32 bytes, and the unused bytes are supplemented with ' \0 ' . |
| CP FirmVer | 3 2 | CP module firmware version number and bootloader version number, the format is the same as the hardware version number; |
| CP _SN Code​ | 1 2 | CP module 12-byte serial number, in ASCII format, with unused bytes supplemented with ' \0 ' ; |
| RFID Mode Ver | 32 | RFID module version number , in ASCII format, ends with ' \0 ' . The actual version number is 2 bytes, less than 32 bytes, and the unused bytes are supplemented with ' \0 ' . The first byte is the main version, and the second byte is the sub-version; for example: Byte0=0x0, Byte1=0x1e, the version number is 3.30. |
| RFIDModeName | 3 2 | RFID module model , in ASCII format, ending with ' \0 ' . The actual model is less than 32 bytes, and the unused bytes are supplemented with ' \0 ' . |
| RFIDMode _SN Code​ | 1 2 | RFID module serial number, in ASCII format. The actual serial number is 4 bytes, less than 32 bytes. The unused bytes are supplemented with ' \0 ' . |

2.2.8 RFM_SET_ ALL_PARAM ( Set Configurable Operating Parameters )

This command is used to set the configurable working parameters of the device . If the user does not set this , the device will use the default settings when working.

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Addr | RFIDPRO | Work Mode | Interface | Baudrate | WGSet | Ant | RfidFreq |
| 0xCF | 0xFF | 0x0071 | 0x19 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 8Bytes |

| PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfidPower | InquiryArea | QValue | Session | AcsAddr | AcsDataLen | FilterTime | TriggerTime | BuzzerTime | Polling Interval |  |
| 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Bytes​ |

  - Addr : The communication address of the device, the default value is 0x00. This address cannot be 0xFF. If it is set to 0xFF, the read/write module will return parameter error information.

  - RFIDPRO: Protocol standard specification of radio frequency RFID equipment , 0x00: ISO 18000-6C; 0x01: GB/T 29768 ; 0x02: GJB 7377.1 ; Currently only ISO is supported 18000-6C.

  - WorkMode : The working mode of the device, the default value is 0 .

| WorkMode | Working Mode |
| --- | --- |
| 0 | Answer Mode |
| 1 | Active Mode |
| 2 | Trigger Mode |

  - Interface: The communication interface of the device. The default value is 0x80 . The specific meanings are as follows:

| Interface | Interface Type |
| --- | --- |
| 0x80 | RS232 |
| 0x40 | RS485 |
| 0x20 | RJ45 |
| 0x10 | WiFi |
| 0x01 | USB |
| 0x02 | keyboard |
| 0x04 | CDC_COM |

  - Baudrate : Serial port baud rate, the default value is 4, the specific meanings are as follows:

| baudrate | Actual baud rate |
| --- | --- |
| 0 | 9600bps |
| 1 | 19200 bps |
| 2 | 38400 bps |
| 3 | 57600 bps |
| 4 | 115200 bps |

  - WGSet : Configuration parameters of the Wiegand data output interface, the default value is 0x00 , the specific meanings are as follows:

| WGSet | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit Definition | 0: Disable Wiegand output 1: Enable Wiegand output | 0: wg26 1: wg34 | 0: Low first 1: High position first | spare | spare | spare | spare | spare |

Wiegand data keyboard output format: Default is 2b 00.

2b 00: Convert directly to decimal characters.

wg26 : 0xA1B2C3 is directly converted to 8-bit decimal characters as ' 10597059 ' ;

wg 34: 0xA1B2C3D4 converts directly to 10 decimal characters as ' 2712847316 ' .

2b 01: Convert to decimal characters by word.

wg26 : 0xA1B2C3 is converted to 8-bit decimal characters by 2H+4H, which is ' 16145763 ' ;

wg 34: 0xA1B2C3D4 is converted to 10 decimal characters as ' 4139450132 ' by pressing 4H+4H .

2b 10: Convert directly to hexadecimal characters.

wg26 : 0xA1B2C3 is directly expressed as 'A1B2C3' using 6-digit hexadecimal ;

wg 34: 0x A1B2C3D4 is directly expressed as 'A1B2C3D4' using 8-bit hexadecimal .

2b 11: Reserved.

  - Ant: All antenna numbers of the device. The bit indicates the antenna to be used. If the corresponding bit value is 1, the antenna is used, and if the value is 0, the antenna is not used. Starting from the lowest bit, the 0th bit indicates antenna No. 1, the 1st bit indicates antenna No. 2, and so on. A maximum of 8 antennas can be represented . Different modules support different antennas , depending on the specific situation. The default value is 0x01 , indicating antenna No. 1 .

  - RfidFreq : RFID frequency-related parameters of the device, used to select the frequency band and the upper and lower frequency limits in each frequency band. The length is 8 bytes. The specific parameters are as follows:

| RfidFreq |  |  |  |  |
| --- | --- | --- | --- | --- |
| R EGION | STRATFREI | STRATFRED | STEPFRE | CN |
| 1 Byte | 2Byte | 2Byte | 2Byte | 1 Byte |

Definition of each byte:

To set the frequency, first determine the frequency band range according to REGION, and then refer to STRATFREI , STRATFRED , STEPFRE , and CN to calculate the specific frequency range.

| Fields | byte | describe |
| --- | --- | --- |
| REGION | 1 | Frequency band ranges in various countries: 0x 00: customized by the user according to needs; 0x01: US [902.75~927.25] 0x02: Korea [917.1~923.5] 0x03: EU [865.1~868.1] 0x 04 ：JAPAN [952.2~953.6] 0x05 : MALAYSIA [ 919.5 ~922.5] 0x06 : EU3 [865.7~867,5 ] 0x07 : CHINA_BAND1 [ 840.125~844.875] 0x08 : C HINA_BAND2 [ 920.125~924.875] |
| STRATFREI | 2 | in megahertz ; for example, 920.125MHz, STRATFREI = 920 = 0x 0 398, high byte = 0x03, low byte = 0x98; |
| STRATFRED | 2 | in megahertz ; for example, 920.125MHz, STRATFRED = 125, high byte = 0x00, low byte = 0x7D |
| STEPFRE | 2 | Frequency step ( KHz ) , please refer to the calculation formula of each frequency band ; for example, 125KHz, STEPFRE = 125, high byte = 0x00, low byte = 0x7D; |
| CN | 1 | Number of channels; |

Frequency calculation formula: 	Minimum frequency: Fmin = STRATFREI + STRATFRED /1000  (Unit: MHz)

Maximum frequency: Fmax = Fmin + STEPFRE *CN/1000 (Unit: MHz)

Calculation formula for each frequency band: (The set frequency range must be within the standards of each country )

Chinese band2: 		Fs = 920.125 + CN * 0.25 (MHz) where CN∈[0, 19].

US band: 				Fs = 902.75 + CN * 0.5 (MHz) where CN∈[0,49].

Korean band: 			Fs = 917.1 + CN * 0.2 (MHz) where CN∈[0, 31].

EU band: 				Fs = 865.1 + CN*0.2(MHz) where CN∈[0, 14].

Ukraine band: 			Fs = 868.0 + CN*0.1(MHz) where CN∈[0, 6].

Peru band: 			Fs = 916.2 + CN *0.9(MHz) where CN∈ [0, 11].

Chinese band1: 		Fs = 840.125 + CN * 0.25 (MHz) where CN∈[0, 19].

EU3 band: 			Fs = 865.7 + CN * 0.6(MHz) where CN∈[0, 3].

US band3: 			Fs = 902 + CN * 0.5 (MHz) where CN∈[0,52].

Taiwan band: 			Fs = 922.25 + CN * 0.5 (MHz) where CN∈[0,11].

  - RfidPower : The RFID output power of the device, in dBm. The value range is [ 0 , 30]dBm. Other values are invalid .

  - InquiryArea : The storage area of the tag that the device wants to access. 0x00: Reserved area; 0x01 (default): EPC storage area ; 0x02: TID storage area; 0x03: USER storage area; 0x04: EPC+TID; 0x05: EPC+USER; 0x06: EPC+TID+USER; other values are reserved. If other values appear in the command , a parameter error message will be returned.

  - QValue : The initial Q value used when querying EPC tags. The Q value should be set to approximately equal 2Q when the number of tags in the field is equal to 2Q. The default value of Q is 4 , and the range of Q is 0 to 15. If other values appear in the command , a parameter error message will be returned.

  - Session: The Session value used when querying the EPC tag. The default value is 0 and the value range is [0, 3]. Other values will return a parameter error message.

| Session | Notes |
| --- | --- |
| 0 | Session uses S0 |
| 1 | Session uses S1 |
| 2 | Session using S2 |
| 3 | Session using S3 |

  - AcsAddr : The starting address of the tag storage area that the device wants to access, unit: Byte, default value: 0x00: When accessing the EPC area, 0x00 indicates the starting address of the EPC number segment of the EPC area except the CRC and PC segments; when accessing other storage areas, 0x00 indicates the starting address of the storage area.

  - AcsDataLen : The data length that the device wants to access the tag storage area, unit: Byte, default value: 0x00.

  - FilterTime : filter time. After a tag data is successfully read, the tags with the same data are filtered out within this time . The unit is S, and the value range is [ 0 , 255 ] . Other values are invalid . The default value is 0, which means no filtering.

  - TriggerTime : The query duration after the device receives the trigger signal, in seconds. The default value is 1. The value range is [ 0 , 255 ] . Other values are invalid .

  - BuzzerTime : The duration of the buzzer after the device is successfully executed. The unit is 10ms. The value range is [ 0 , 255 ] . Other values are invalid . The default value is 1. When it is 0, it means that the buzzer does not sound.

  - PollingInterval : polling interval, unit: 10ms, value range: [ 0 , 255 ] , other invalid , default is 1 .

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0xXX​​ | 0x00 71 | 0x01 | 1 Byte | 2Byte |

  - STATUS:

  - 0x00: command executed successfully;

  - 0x01: parameter error ;

  - Other values: invalid;

2.2.9​ RFM_GET_ ALL_PARAM ( Read configurable operating parameters )

This command is used to set the configurable working parameters of the device .

- Format

| HEAD | ADDR​ | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF​ | 0x00 72 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STATUS | Addr | RFIDPRO | Work Mode | Interface | Baudrate | WGSet | Ant | RfidFreq |
| 0xCF | 0xFF | 0x0071 | 0x1A | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 8Bytes |

| PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfidPower | InquiryArea | QValue | Session | AcsAddr | AcsDataLen | FilterTime | TriggerTime | BuzzerTime | Polling Interval |  |
| 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Bytes​ |

  - STATUS:

  - 0x00: command executed successfully;

  - Other values: invalid ;

  - For other parameters, refer to RFM_SET_ALL_PARAM.

2.2.10​​ RFM_SET_ G ET_IOPUT _PARAM ( Set read input and output control )

This command is used to read the input and output control parameters of the device.

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN | OPTION | Relay EN | Relay Time | Relay Power EN | Trig Way | Buffer EN | Protocol EN | Protocol Type | Protocol Format |  |
| 0xCF | 0xFF | 0x0074 | 0x12 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 10 Bytes | 2Bytes |

Parameter explanation:

Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

Relay EN : Relay enable parameter, length 1 byte , default 0x00 .

bit0: 1 : Relay 1 is enabled, 0: Relay 1 is not enabled .

bit1: 1 : Relay 2 is enabled, 0: Relay 2 is not enabled .

RelayTime : Relay normally open foot conduction time parameter, length 1 byte, default 0x03 .

RelayPowerEN : relay common terminal power on enable parameter , length 1 byte , default 0x00 . 0x01 is on , 0x00 is off .

TrigWay : Valid trigger mode parameter of the trigger pin, 1 byte in length , default value is 0x00 .

0x01 is triggered by high level (2V-5V), and 0x00 is triggered by low level (less than 0.5V).

BufferEN : Buffer function enable parameter , length 1 byte, default 0x00 . 0x01 enables, 0x00 disables.

ProtocolEN : Specific data exchange function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

ProtocolType : Specific data exchange protocol type, length 1 byte. Default 0x00 .

Modbus-RTU:0x00,ASCII:0x01,HEX:0x02,Modbus-TCP:0x03.

ProtocolFormat : Specific data exchange protocol format, length 10 bytes . Default [0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00] .

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0073 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN | STATUS | OPTION | GPO EN | Reserve | GPOWM | GPIWM | Reserve | Protocol EN | Protocol Type | Protocol Format |  |
| 0xCF | 0xFF | 0x0074 | 0x13 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 10 Bytes | 2 Bytes​ |

  - STATUS:

  - 0x00: command executed successfully;

  - Other values: invalid ;

2.2.1 1 RFM_SET_GET_WiFi_PARAM ( Set / Read WiFi information )

This command sets/reads the module's WiFi information. If it is set as TCP Client, the corresponding remote IP address and port need to be set.

Wi-Fi network configuration mode: 1. SmartConfig configures the network through the mobile phone app, 2. Serial port configuration, configures by sending ssid and pwd . After successful configuration, it will automatically connect after power on.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | WiFiPara |  |
| 0xCF | 0xFF | 0x0075 | 1 Byte | 1 Byte | NC or 1Byte | 2Byte |

  - Option: command control option

0x01: Setting;

0x02: read;

Other values: invalid;

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0075 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x00 | 0x0075 | 1 Byte | 1 Byte | 1 Byte | NByte | 2Byte |

| Fields |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: Command execution successful; 0x01: Parameter error. |
| Option |  | 1 | 0x01: Set; 0x02: Get |
| WiFiPara | WiFiEN | 1 | WiFi enabling parameters, mode selection. bit0: 0: turn off wifi ; 1: turn on wifi bit1: 0: smartconfig 1: serial port network configuration. Other bits default to 0; |
|  | SSID | 32 | Target WiFi name, ASIC code. Invalid bytes are filled with 0x00 |
|  | PWD | 64 | Target WiFi password, ASIC code. Invalid bytes are filled with 0x00 |
|  | RemoteIPAddr | 4 | Remote IP address. If the IP address is 192.168.1.1, the data is [0xC0, A8, 0x01, 0x01] |
|  | RemotePort | 2 | Remote IP port number, high byte first, low byte last |

2.2.1 2​ RFM_ S ET_GET_S-PERMISSION _PARAM ( Set read permission )

This command is used to set password and mask query permission parameters, relay usage parameters, cache switch parameters and public protocol usage parameters.

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0073 | 0x29 | 1Byte | 1Byte | 4Bytes | 1 Byte | 1 Byte | 1 Byte | 31Bytes​ | 1 Byte | 2 Bytes​ |

Parameter explanation:

Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

CodeEN : Password function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

Codes: Access code of the tag, 4 bytes in length. Default value is [0x00,0x00,0x00,0x00] .

MaskEN : Mask function enable parameter, length 1 byte. 0x01 enables, 0x00 disables. Default 0x00 .

StartAdd : mask start address, length 1 byte, unit byte. Default 0x00 .

Mask Len: Mask length, 1 byte, unit byte, maximum 31. Default 0x00 .

MaskData : Mask data, 31 bytes in length. When the mask length is less than 31 bytes, the following bytes are padded with 0.

The default is all 0 .

MaskCondition : Mask condition, length 1 byte . 0x00: password or mask meets the requirement; 0x01: password and mask meet the requirement at the same time. Default is 0x00 .

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0073 | 1Byte | 1Byte | 1Byte | 2Byte |

获取响应：

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0073 | 0x29 | 1 Byte | 1 Byte | 0xFF | 4 0xFF | 1 Byte | 1 Byte | 1 Byte | 31Bytes​ | 1 Byte | 2 Bytes​ |

  - STATUS:

1) 0x00: command executed successfully;

- Other values: invalid ;

2.2.1 3 RFM_RELEASE_CLOSE_RELAY1 ( Release /Close Relay 1)

This command is used to release and close the relay.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | Valid'Time |  |
| 0xCF | 0xFF | 0x0077 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

  - Option: command control option

0x01: Release;

0x02: Closed;

Other values: invalid;

  - ValidTime : The effective time when closed, in seconds, with a default value of 1. When the ValidTime value is equal to 0, relay 1 will remain in the closed state; when the ValidTime value is greater than 0, relay 1 will be released after closing for ValidTime seconds.

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | Option | Valid'Time | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0077 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Byte |

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: Command execution successful; 0x01: Parameter error. |
| Option | 1 | 0x01: Release; 0x02: Close |
| ValidTime | 1 | The effective time when closed, in seconds, the default value is 1. |

2.2.1 4 RFM_RELEASE_CLOSE_RELAY2 ( Release /Close Relay 2)

This command is used to release and close the relay.

- Format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | Valid'Time |  |
| 0xCF | 0xFF | 0x0078 | 1 Byte | 1 Byte | 1 Byte | 2Byte |

  - Option: command control option

0x01: Release;

0x02: Closed;

Other values: invalid;

  - ValidTime : The effective time when closed, in seconds, with a default value of 1. When the ValidTime value is equal to 0, Relay 2 will remain in the closed state; when the ValidTime value is greater than 0, Relay 2 will be released after closing for ValidTime seconds.

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | Option | Valid'Time | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0078 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Byte |

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: Command execution successful; 0x01: Parameter error. |
| Option | 1 | 0x01: Release; 0x02: Close |
| ValidTime | 1 | The effective time when closed, in seconds, the default value is 1. |

2.2.15​​ RFM_ S ET_GET_AntN -RSSI-Filter ( set the RSSI filter threshold for reading each antenna port )

This command is used to filter tags with different signal strengths . RSSI is in negative form, and the larger the absolute value, the farther the tag is.

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN | OPTION | BasicRSSI | AntN-RSSIData |  |
| 0xCF | 0xFF | 0x007 9 | 0x13 | 1 Byte | 2Byte | Ant1-RSSIData~Ant16-RSSIData 16Byte | 2 Bytes​ |

Parameter explanation:

- Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

- BasicRSSI : The basic value of the RSSI filter threshold of each antenna port , 2 bytes . The value range is [-32767, 32767]. Negative numbers are in the form of two's complement. The default value is 0xFFBF: -65.

- AntN-RSSIData : RSSI filter threshold offset value of each antenna port , 16 bytes . The RSSI filter threshold offset value of each antenna port occupies 1 byte, with a value range of [0, 255] and a default value of 0.

Note: The RSSI filter value for each antenna port X is BasicRSSI - AntX-RSSIData . For example: BasicRSSI is -65, Ant1-RSSIData is 1, then the RSSI filter value for antenna 1 is -66. When the RSSI filter value is less than -174, such as -175, the filter function will not work.

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0079 | 0x02 | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | Control Field |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN | STATUS | OPTION | BasicRSSI | AntN-RSSIData |  |
| 0xCF | 0xFF | 0x00 79 | 0x14 | 1 Byte | 1 Byte | 2Byte | Ant1-RSSIData~Ant16-RSSIData 16Byte | 2 Bytes​ |

  - STATUS:

1) 0x00: command executed successfully;

- Other values: invalid ;

2.3 International standard (ISO 18000-6 C ) protocol related commands

2.3.1 RFM_INVENTORYISO_CONTINUE ( Tag Inventory )

This command starts the international standard multi-tag inventory command, and the inventory function has an anti-collision algorithm.

This command can specify the duration of the inventory count. If the input inventory count count is 0, it means that the inventory will continue until the stop inventory count command is received. During the inventory count process, each time a tag is successfully counted, the command response with a "STATUS" value of "0" will return the information of the newly counted tag.

When the inventory command is successfully executed, the command response with a "STATUS" value of "0x12" will be returned to notify that the inventory command has been executed.

- Format

| HEAD | ADDR​ | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | InvType | InvParam |  |
| 0xCF | 0xFF​ | 0x0001 | 0x05 | 1 Byte | 4Byte | 2Byte |

  - InvType : Inventory method:

0x00: count labels by time, stop counting after executing the specified time or after receiving the stop counting command;

0x 01: Inventory is performed according to the number of cycles. The inventory is stopped after the specified number of polls is executed or after receiving the stop inventory command;

  - InvParam : Inventory method parameters:

1. If InvType is 0x00:

InvParam indicates the inventory time in seconds. If the value is 0, it means that the inventory will continue until a stop command is received.

2. If InvType is 0x01:

InvParam indicates the number of inventory counts , in times , and the value must be greater than 0;

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | RSSI | Antenna | Channel | EPC LEN | EPC NUM |  |
| 0xCF | 0x00​ | 0x0001 | 1 Byte | 1 Byte | 2Bytes | 1 Byte | 1 Byte | 1 Byte | N Bytes | 2Byte |

Definition of each byte:

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: The tag is successfully counted and the tag information is included in PAYLOAD; 0x01: Q, MemBank parameter value is incorrect or Length is inconsistent with Mask data length; 0x02: Command execution failed due to an internal module error; 0x12: No tag is counted or the entire count command is executed; 0x17: The tag data exceeds the maximum transmission length of the serial port; Other values: invalid; |
| RSSI | 2 | RSSI of tag ACK response, in dBm, signed number, negative number uses two's complement format; |
| Channel | 1 | The channel from which the tag data is received. The value starts from 0, where 0 represents channel 0, 1 represents channel 1, and so on. |
| EPC LEN | 1 | The length of the tag's EPC number (bytes) ; |
| EPC NUM | N Byte | The EPC number of the label ; |

2.3.2 RFM_INVENTORY_STOP ( Stop Inventory )

This command allows the user to actively stop the international standard multi-tag anti-collision inventory process.

- Format

| HEAD | ADDR​ | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF​ | 0x00 02 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 02 | 0x01 | 1 Byte | 2Byte |

 STATUS: This command will only return a successful execution (value 0x00), other values are invalid;

2.3.3 RFM_READISO_TAG ( Read tag data )

This command frame is for the module to read the data in the international standard protocol tag storage area.

Before calling this method, please set the inventory condition (for selecting tags) through the RFM_SETISO_SELECTMASK command. When performing international standard protocol tag data operations (reading tag data, writing tag data, locking tags, and deactivating tags), the tag will return the tag operation status. At this time, the module's response contains a byte of tag operation status. The definition of tag operation status is shown in Appendix A.

- Format

| HEAD | ADDR​ | CMD | LEN |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | AccPwd | MemBank | WordPtr | WordCount |  |
| 0xCF | 0xFF​ | 0x00 03 | 1 Byte | 1 Byte | 4 Bytes | 1 Byte | 2 Bytes | 1 Bytes | 2Byte |

Definition of each field:

| Fields | byte | describe |
| --- | --- | --- |
| Option | 1 | Command options (this field is not used, invalid , default value is 0x00 ) ; |
| AccPwd | 4 | Access password, used for the tag to enter the secure state, the default value is 0x00000000; |
| MemBank | 1 | The storage area of the tag to be read, the value list is as follows : 0x00: Reserved; 0x01: EPC; 0x02: TID; 0x03: User; |
| WordPtr | 2 | (word ) of the logical storage area ; |
| WordCount | 1 | The number of words to be read cannot be 0. The default value is 4. The value range is [1,120] |

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | PAYLOAD |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM | WordCount | Data |  |
| 0xCF | 0x00 | 0x0003 | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte | 2Bytes | 1Byte | N Bytes | 1Bytes | N-Byte | 2Byte |

Definition of each field:

based on STATUS and TagStatus .

| Fields | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | From which antenna is the reading |
| CRC | 2 | CRC data in the tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes) ; |
| EPC NUM | N Byte | The EPC number of the label ; |
| WordCount | 1 | The number of tag data words successfully read ; |
| Data | N Byte | The tag data successfully read has a length of WordCount×2 bytes; |

2.3.4​​ RFM_SETISO_SELECTMASK ( Select the tag to be operated )

EPC number required for tag operation (read, write, lock and deactivate) .

- Format

| HEAD | ADDR​ | CMD | LEN |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pointer | Length | Mask |  |
| 0xCF | 0xFF​ | 0x00 07 | 1 Byte | 2Bytes | 1 Byte | N-Bytes | 2Byte |

  - Pointer: reserved, the default value is 0x0000;

  - The bit length of the EPC number to be matched . The default value is 0x00 .

  - Mask: data to be matched. The effective data length is Length bits. If Length is an odd number, the low bits of the mask need to be padded with 0.

- Response format and status byte

| HEAD | ADDR​ | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00​ | 0x00 07 | 0x01 | 1 Byte | 2Byte |

  - STATUS:

  - 0x00: command executed successfully;

  - 0x01: parameter error;

  - Other values: invalid;

2.3.5​ RFM_SET_SELPRM ( Set Select command parameters )

This command is used to set the Select instruction parameters of the module's specified protocol, including " MemBank ", "Target", "Action", "Pointer", "Length", and "Mask". For the specific application and meaning of each parameter, please refer to the protocol description.

- Format

| HEAD | CMD | LEN |  |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protocol | Target | Action | Membank | Pointer | Length | Mask |  |
| 0xCF | 0x00 10 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2Bytes | 1 Byte | N-Bytes | 2 Byte |

  - Protocol : Protocol type

0x00 : ISO

  - Target: Select to modify the target.

0x00 : S0 ; 0x01 : S1 ; 0x02 : S2 ; 0x03 : S3 ; 0x04 : SL ; 0x05-0xFF : reserved .

  - Action: Select the matching condition rule

| Action | ISO |
| --- | --- |
| 0x00 | If the conditions are met, set it to SL or A; if the conditions are not met, set it to non-SL or B |
| 0x01 | Eligible conditions are set to SL or A |
| 0x02 | If the conditions are not met, set to non-SL or B |
| 0x03 | Meet the conditions to reverse SL or A->B, B->A |
| 0x04 | If the conditions are met, it is set to non-SL or B; if the conditions are not met, it is set to SL or A |
| 0x05 | Eligible conditions are set to non-SL or B |
| 0x06 | If the conditions are not met, set to SL or A |
| 0x07 | Does not meet the conditions to reverse SL or A->B, B->A |

  - MemBank : The storage area where the selected and matched data is located

| Membank | ISO | Interpretation | Remark |
| --- | --- | --- | --- |
| 0x00 | RFU | Label information area | Label information area |
| 0x10 | EPC | Coding region | Coding region |
| 0x20 | TID | Safe Zone | Safe Zone |
| 0x30 | User | User Area | User subarea 0 |

  - Pointer: The starting address of the match, in bits

  - Length: The length of the matching mask, in bits

  - Mask: data to be matched. The effective data length is Length bits. If Length is an odd number, the lower bits of the mask need to be padded with 0.

- Response format and status byte

| HEAD | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Protocol |  |
| 0xCF | 0x00 10 | 1 Byte | 1 Byte | 1 Byte | 2 Byte |

  - Status: command response code

0x00: command executed successfully;

0x01: parameter error;

Other values: invalid;

  - Protocol : Protocol type

0x00 : ISO

2.3.6​ RFM_GET_SELPRM ( Get Select command parameters )

This command is used to obtain the Select/Sort instruction parameters of the specified protocol of the module, including " MemBank ", "Target", "Action", "Pointer", "Length", and "Mask". For the specific application and meaning of each parameter, please refer to the protocol description.

- Format

| HEAD | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- |
|  |  |  | Protocol |  |
| 0xCF | 0x00 11 | 1 Byte | 1 Byte | 2 Byte |

  - Protocol : Protocol type

0x00 : ISO

- Response format and status byte

| HEAD | CMD | LEN |  |  |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Protocal | Target | Action | Membank | Pointer | Length | Mask |  |
| 0xCF | 0x0011 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes | 1Byte | N-Bytes | 2Byte |

  - Status：命令响应码

0x00: command execution successful; 0x01: parameter error; other values: invalid;

  - Protocol : Protocol type

0x00 : ISO

  - Target: Select to modify the target.

0x00 : S0 0x01 : S1 0x02 : S2 0x03 : S3 0x04 : SL 0x05-0xFF : Reserved

  - Action: Select the matching condition rule

| Action | ISO |
| --- | --- |
| 0x00 | If the conditions are met, set it to SL or A; if the conditions are not met, set it to non-SL or B |
| 0x01 | Eligible conditions are set to SL or A |
| 0x02 | If the conditions are not met, set to non-SL or B |
| 0x03 | Meet the conditions to reverse SL or A->B, B->A |
| 0x04 | If the conditions are met, it is set to non-SL or B; if the conditions are not met, it is set to SL or A |
| 0x05 | Eligible conditions are set to non-SL or B |
| 0x06 | If the conditions are not met, set to SL or A |
| 0x07 | Does not meet the conditions to reverse SL or A->B, B->A |

  - MemBank : The storage area where the selected and matched data is located

| Membank | ISO | Interpretation | Remark |
| --- | --- | --- | --- |
| 0x00 | RFU | Label information area | Label information area |
| 0x10 | EPC | Coding region | Coding region |
| 0x20 | TID | Safe Zone | Safe Zone |
| 0x30 | User | User Area | User subarea 0 |

  - Pointer: The starting address of the match, in bits

  - Length: The length of the matching mask, in bits

  - Mask: data to be matched. The effective data length is Length bits. If Length is an odd number, the lower bits of the mask need to be padded with 0.

2.3.8​ RFM_GET_QUERY_PARAM ( Get Query command parameters )

This command is used to obtain the Query instruction parameters of the specified protocol of the module, including "Sel", "Session", and "Target". For the specific application and meaning of each parameter, please refer to the protocol description .

- Format

| HEAD | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- |
|  |  |  | Protocol |  |
| 0xCF | 0x00 13 | 1 Byte | 1 Byte | 2 Byte |

  - Protocol : Protocol type

0x00 : ISO

- Response format and status byte

| HEAD | CMD | LEN |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Protocol | Sel | Session | Target |  |
| 0xCF | 0x00 13 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2 Byte |

  - Status: command response code

0x00: command execution successful; 0x01: parameter error; other values: invalid;

  - Protocol : Protocol type

0x00 : ISO

  - Sel: Specifies the label of the inventory response

0x00 : ALL; 0x01:ALL; 0x02: ~SL; 0x03: SL .

  - Session: Specifies the section of the inventory label. This field is valid for ISO and national standards, and is not used for national standards.

0x00: S0 ; 0x01: S1 ; 0x02: S2 ; 0x03: S3

  - Target : Inventory identification

0x00: A

0x01: B

2.3.7​ RFM_SET_QUERY_PARAM ( Set Query command parameters )

This command is used to set the Query instruction parameters of the specified protocol of the module, including "Sel", "Session", and "Target". For the specific application and meaning of each parameter, please refer to the protocol description.

- Format

| HEAD | CMD | LEN |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protocol | Sel | Session | Target |  |
| 0xCF | 0x00 12 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2 Byte |

  - Protocol : Protocol type

0x00 : ISO

  - Sel: Specifies the label of the inventory response

0x00 : ALL; 0x01:ALL; 0x02: ~SL; 0x03: SL .

  - Session: Specifies the section of the inventory tag

0x00: S0 ; 0x01: S1 ; 0x02: S2 ; 0x03: S3

  - Target : Inventory identification

0x00: A

0x01: B

- Response format and status byte

| HEAD | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Status | Protocol |  |
| 0xCF | 0x00 12 | 0x02 | 1 Byte | 1 Byte | 2 Byte |

  - Status: command response code

0x00: command execution successful; 0x01: parameter error; other values: invalid;

  - Protocol : Protocol type

0x00 : ISO

## 2.4 GPIO Control Commands

2.4.1 RFM_SET_GET_GPIO_WORKPARAM ( Set / get GPIO working parameters )

Set/get GPIO working parameters .

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | GPIOMODE | GPIEN | INTLEVEL | GPOEN | PUTLEVEL | PUTTIME |  |
| 0xCF | 0xFF | 0x0080 | 0x0E | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 8 Bytes​ | 2 Bytes​ |

Parameter explanation:

- Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

- GPIOMODE: GP IO working mode parameter, length 1 byte, default 0x 00 .

| GPIOMODE | Interpretation |
| --- | --- |
| 0x00 | General Mode: In this mode, the GPIX input valid level triggers the card reading, and the card reading triggers the GPOX output valid level. The duration of GPOX output is determined by the GPOX_Put_Time of GPOX in PUT TIME . In this mode, when GPOX_Put_Time = 0xFF, GPOX automatically and continuously outputs a valid level, and then GPOX enters the instruction control output mode. |
| 0x01 | Door access mode: In this mode, GPIO works according to the settings in the gate channel working parameters. For specific explanations of the gate channel working parameters, see 2.6.1. |
| other | reserve. |

- GPIEN: GPI enable status parameter, length 1 byte, default 0x 00. The enable status length of each GPI is 1 bit. 0: Disable; 1: Enable.

8bit interpretation of GPIEN :

| bit | Interpretation |
| --- | --- |
| 7 | GPI8 Enable Status |
| 6 | GPI7 Enable Status |
| 5 | GPI6 Enable Status |
| 4 | GPI5 Enable Status |
| 3 | GPI4 Enable Status |
| 2 | GPI3 Enable Status |
| 1 | GPI2 Enable Status |
| 0 | GPI1 Enable Status |

- IUTLEVEL :

GPI valid level parameter, length 1 byte, default 0x 00 .

The effective level parameter length of each GPI is 1 bit. 0: low level; 1: high level.

8bit interpretation of IUTLEVEL :

| bit | Interpretation |
| --- | --- |
| 7 | GPI8 effective level |
| 6 | GPI7 effective level |
| 5 | GPI6 effective level |
| 4 | GPI5 effective level |
| 3 | GPI4 effective level |
| 2 | GPI3 effective level |
| 1 | GPI2 effective level |
| 0 | GPI1 effective level |

- GPOEN: GPO output enable status parameter, length 1 byte, default 0x 00. The output enable status length of each GPO is 1 bit . 0: Disable; 1: Enable.

8-bit interpretation of GPOEN :

| bit | Interpretation |
| --- | --- |
| 7 | GPO8 output enable status |
| 6 | GPO7 output enable status |
| 5 | GPO6 output enable status |
| 4 | GPO5 output enable status |
| 3 | GPO4 output enable status |
| 2 | GPO3 output enable status |
| 1 | GPO2 output enable status |
| 0 | GPO1 output enable status |

- PUTLEVEL :

GPO output valid level parameter, length 1 byte, default 0x 00 .

effective level parameter length of each GPO is 1 bit. 0: low level; 1: high level.

8bit interpretation of PUTLEVEL :

| bit | Interpretation |
| --- | --- |
| 7 | GPO8 output valid level |
| 6 | GPO7 output valid level |
| 5 | GPO6 output effective level |
| 4 | GPO5 output valid level |
| 3 | GPO4 output valid level |
| 2 | GPO3 output valid level |
| 1 | GPO2 output valid level |
| 0 | GPO1 output valid level |

- PUTTIME :

GPO output valid level duration, length 8 bytes. Specific interpretation is as follows:

| name | length | Interpretation | Notes |
| --- | --- | --- | --- |
| GPO1_Put_Time | 1 Bytes | GPOX_Put_Time is the duration of GPOX outputting a valid level , in units of 100mS. General output mode: When GPIOMODE is in general mode, GPOX is triggered by GPIX to control the effective level of the response output, with a value range of 0~254 and a default of 30. If the card is not read again within the response output time , the response output ends ; if the card is read again within the response output time , the response output time is reset to GPOX_Put_Time *100mS. Command control output mode: When GPIOMODE is in general mode and GPOX_Put_Time = 255 (0xFF), GPOX automatically and continuously outputs a valid level. At this time, GPOX enters the instruction control output mode. Inventory output mode: When GPIOMODE is gate channel mode and GATEMODE is inventory mode, GPOX responds to the output valid level controlled by the card reading status, GPOX_Put_Time is invalid and defaults to 0 . EAS output mode: When GPIOMODE is gate channel mode and GATEMODE is EAS mode, GPOX is triggered by EAS event to control the response output valid level, the value range is 0~255, the default is 3. |  |
| GPO2_Put_Time | 1 Bytes |  |  |
| GPO3_Put_Time | 1 Bytes |  |  |
| GPO4_Put_Time | 1 Bytes |  |  |
| GPO5_Put_Time | 1 Bytes |  |  |
| GPO6_Put_Time | 1 Bytes |  |  |
| GPO7_Put_Time | 1 Bytes |  |  |
| GPO8_Put_Time | 1 Bytes |  |  |

- Response format and status byte

Set the response:

| HEAD | Control Field |  |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION |  |
| 0xCF | 0x00 | 0x00 8 0 | 0x02 | 1 Byte | 1Byte | 2Bytes |

获取响应：

| HEAD | Control Field |  |  | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN |  |  |
| 0xCF | 0x00 | 0x0080 | 0x0F | 15Bytes | 2Bytes |

| PAYLOAD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| STATUS | OPTION | GPIOMODE | GPIEN | INTLEVEL | GPOEN | PUTLEVEL | PUTTIME |
| 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 8 Bytes​ |

STATUS:

0x00: command executed successfully;

Other values: invalid ;

2.4.2 RFM_GET_GPIO_LEVELS ( Get GPIO input and output levels )

Get the GPI input level, GPO output level and system time .

- Format

| HEAD | Control Field |  |  | CHECK |
| --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN |  |
| 0xCF | 0x00 | 0x00 8 1 | 0x0 0 | 2 Bytes​ |

- Response format and status byte

Get the response:

| HEAD | Control Field |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | GPILEVEL | GPOLEVEL | SYSTEAMTIME |  |
| 0xCF | 0x00 | 0x00 8 1 | 0x0 7 | 1 Byte | 1 Byte | 1 Byte | 4Bytes | 2 Bytes​ |

- STATUS: 0x00: Command executed successfully; others are invalid.

- GPILEVEL: Real-time input level of 8 GPIs, length 1 byte.

8-bit interpretation of GPILEVEL:

| bit | Interpretation |
| --- | --- |
| 7 | GPI8 Input Levels |
| 6 | GPI7 Input Levels |
| 5 | GPI6 Input Level |
| 4 | GPI5 Input Levels |
| 3 | GPI4 Input Level |
| 2 | GPI3 Input Level |
| 1 | GPI2 Input Level |
| 0 | GPI1 input level |

If the bit is 0, it means that the input of the GPI is at a low level; if the bit is 1, it means that the input of the GPI is at a high level.

- GPOLEVEL: Real-time output level of 8 GPOs, length 1 byte.

8-bit interpretation of GPOLEVEL:

| bit | Interpretation |
| --- | --- |
| 7 | GPO8 input level |
| 6 | GPO7 input level |
| 5 | GPO6 input level |
| 4 | GPO5 input level |
| 3 | GPO4 input level |
| 2 | GPO3 input level |
| 1 | GPO2 input level |
| 0 | GPO1 input level |

When the bit is 0, it means the output of the GPO is low level; when the bit is 1, it means the output of the GPO is high level.

- SYS TIME: system time when uploading data, length 4 bytes , high byte first, unit ms . For example, 0x 12345678 , the actual system time is: 305419896 ms.

2. 5 Door access control commands

The following commands are owned by the door channel alone, inventory mode, and EAS alarm mode.

2.5.1 RFM_GET_GATE_STATUS ( Get GATE Status )​​​​​​

Get the status of GATE and upload the trigger condition information.

- Active query command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x008 2 | 0x00 | 2Byte |

Parameter explanation:

- Response format and status byte

response:

| HEAD | Control Field |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN | STATUS | GATEDIR | RECV | SYS TIME |  |
| 0xCF | 0xFF | 0x00 8 2 | 0x26 | 1 Byte | 1 Byte | 32Byte | 4byte | 2 Bytes​ |

  - STATUS:

- 0x00: command executed successfully;

- Other values: invalid ;

- GATEDIR:

0x00: no action; 0x10: input; 0x20: output ; 0x50: EAS triggered; 0x51: EAS not triggered .

- RECV: reserved , 32 bytes.

- SYS TIME: System time when uploading data, in ms .

4 bytes , high byte first, low byte last. For example, 0x 12345678 , the actual system time is: 305419896 ms

- GATEGPI triggers the upload message format

Used to remind the host computer to trigger GATEGPI and record the entry and exit of personnel;

| HEAD | Control Field |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | GATEDIR | CUSTOMERINFO | SYS TIME |  |
| 0xCF | 0xFF | 0x0082 | 0x07 | 1Byte | 1Byte | 32Byte | 4byte | 2Bytes |

  - OPTION：

- 0x0 1 : Actively report status changes ;

- Other values: invalid ;

- GATEDIR:

0x00: No action; at this time, there is no tag data behind, and CUSTOMERINFO is all 0.

0x10: In; at this time, there is no label data behind, and CUSTOMERINFO is all 0.

0x20: Out ; at this time, there is no label data behind, and CUSTOMERINFO is all 0.

0x50: EAS is triggered; at this time, there is tag data behind.

0x51: EAS is not triggered; at this time, there is tag data behind.

- CUSTOMERINFO: Customer code information, 32 bytes in length, included in PAYLOAD only when the card is read.

| CUSTOMERINFO |  |
| --- | --- |
| CODELEN | CODENUM |
| 1Byte, customer code length | 31 Bytes, customer code number. If the number is less than 31 Bytes, zeros will be added after the number. |

- SYS TIME: System time when uploading data, in ms .

4 bytes , high bit first, for example 0x12345678 , the actual system time is: 305419896 ms.

2.5.2 RFM_SET_GET_GATE_WORKPARAM ( Set / Get Gate Channel Working Parameters )​

Set/get door channel working parameters: channel working mode, channel detection GPI, EAS working parameters and response GPO.

- Set the command format

| HEAD | Control Field |  |  | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN |  |  |
| 0xCF | 0xFF | 0x00 8 3 | 0x12 | 1 8 Bytes​ | 2 Bytes​ |

| PAYLOAD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OPTION | GATE MODE | GATE GPI1 | GATE GPI2 | POWER GPO | READING GPO | RECV GPIO | EAS MODE | EAS GPO | RECV FUNC |
| 0x01 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 4Byte | 1 Byte | 1 Byte | 6 Byte |

Parameter explanation:

- Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

- GATEMODE :  Channel gate working mode parameter, length 1 byte, default 0x00.

0x00: Inventory mode: inventory counting, etc.

0x01: EAS mode: scenarios such as personnel entry and exit.

- GATEGPI1: Gate channel 1 detection GPI parameters, length 1 byte, default 0x00.

8-bit interpretation of GATEGPI1:

| bit | Interpretation |
| --- | --- |
| 7 | GPI8 is the GPI for gate channel 1 |
| 6 | GPI7 is the GPI for gate channel 1 |
| 5 | GPI6 is the GPI for gate channel 1 |
| 4 | GPI5 is the GPI for gate channel 1 |
| 3 | GPI4 is the GPI for gate channel 1 |
| 2 | GPI3 is the GPI for gate channel 1 |
| 1 | GPI2 is the GPI for gate channel 1 |
| 0 | GPI1 is the GPI for gate channel 1 |

- GATEGPI2: Gate channel 2 detection GPI parameters, length 1 byte, default 0x00.

8bit interpretation of GATEGPI2:

| bit | Interpretation |
| --- | --- |
| 7 | GPI8 is the GPI for gate channel 2 |
| 6 | GPI7 is the GPI for gate channel 2 |
| 5 | GPI6 is the GPI for gate channel 2 |
| 4 | GPI5 is the GPI for gate channel 2 |
| 3 | GPI4 is the GPI for gate channel 2 |
| 2 | GPI3 is the GPI for gate channel 2 |
| 1 | GPI2 is the GPI for gate channel 2 |
| 0 | GPI1 is the GPI for gate channel 2 |

- POWERGPO: Power-on indication GPO in gate channel inventory mode, length 1 byte, default 0x01.

8-bit interpretation of POWERGPO:

| bit | Interpretation |
| --- | --- |
| 7 | GPO8 is the power-on indication GPO |
| 6 | GPO7 is the power-on indication GPO |
| 5 | GPO6 is the power-on indication GPO |
| 4 | GPO5 is the power-on indication GPO |
| 3 | GPO4 is the power-on indication GPO |
| 2 | GPO3 is the power-on indication GPO |
| 1 | GPO2 is the power-on indication GPO |
| 0 | GPO1 is the power-on indication GPO |

In the door channel inventory mode, this parameter indicates that GPOX responds to GPO for power-on indication. When powered on and not reading a card, GPOX outputs the effective level (determined by GPOX working parameters); when reading a card, GPOX outputs the reverse level of the effective level, and exits the card reading state. GPOX will maintain the output reverse level for 300mS, and then switch to outputting the effective level.

- READINGGPO: card reading indication GPO in door channel inventory mode, length 1 byte, default 0x02.

8-bit interpretation of READINGGPO:

| bit | Interpretation |
| --- | --- |
| 7 | GPO8 is the card reading indication GPO |
| 6 | GPO7 is the card reading indication GPO |
| 5 | GPO6 is the card reading indication GPO |
| 4 | GPO5 is the card reading indication GPO |
| 3 | GPO4 is the card reading indication GPO |
| 2 | GPO3 is the card reading indication GPO |
| 1 | GPO2 is the card reading indication GPO |
| 0 | GPO1 is the card reading indication GPO |

In the door channel inventory mode, this parameter indicates that GPOX responds to GPO for card reading indication. When reading a card, GPOX outputs a valid level (determined by GPOX working parameters); when reading a card, GPOX outputs a PWM wave, and a smaller PWM wave period indicates a faster card reading speed; when not reading a card, GPOX outputs a reverse level of the valid level.

- RECVGPIO: Reserved parameters, 4 bytes.

- EASMODE: EAS working parameters, length 1 byte, default 0x00.

8bit interpretation of EASMODE:

| bit | Interpretation |
| --- | --- |
| 7~4 | EAS matching method: 0000: Specific data match. When reading the card, it is necessary to determine whether the data at the specified address of the tag matches. 0001: Specific tag matching, only tags with specific matching marks will respond when reading the card. |
| 3~0 | EAS response mode: 0000: Matching successful response. 0001: Matching failure response |

- EASGPO: Gate channel EAS mode response GPO, length 1 byte, default 0x01.

When the gate channel is in EAS mode, this parameter indicates that GPOX responds to GPO in EAS mode. When EAS is triggered, GPOX outputs a valid level (determined by GPOX working parameters); when EAS is not triggered, GPOX outputs the reverse level of the valid level.

8-bit interpretation of EASGPO:

| bit | Interpretation |
| --- | --- |
| 7 | GPO8 is the gate channel EAS mode response GPO |
| 6 | GPO7 is the GPO for the gated EAS mode response |
| 5 | GPO6 is the GPO for the gated EAS mode response |
| 4 | GPO5 is the Gated Channel EAS Mode Response GPO |
| 3 | GPO4 is the door channel EAS mode response GPO |
| 2 | GPO3 is the Gated Channel EAS Mode Response GPO |
| 1 | GPO2 is the Gated Channel EAS Mode Response GPO |
| 0 | GPO1 is the gate channel EAS mode response GPO |

- RECVFUNC: Reserved parameters, 6 bytes.

- Get command format

| HEAD | Control Field |  |  | OPTION | CHECK |
| --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN |  |  |
| 0xCF | 0xFF | 0x00 8 3 | 0x01 | 0x02 | 2 Bytes​ |

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | OPTION | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x00 8 3 | 0x02 | 1 Byte | 1 Byte | 2Byte |

Get the response:

| HEAD | Control Field |  |  | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN |  |  |
| 0xCF | 0x 00 | 0x00 8 3 | 0x 09 | 11 Bytes​ | 2 Bytes​ |

| PAYLOAD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STATUS | OPTION | GATEMODE | GATEGPI1 | GATEGPI2 | POWERGPO | READINGGPO | EASMODE | EASGPO |
| 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte |

  - STATUS:

- 0x00: command executed successfully;

- Other values: invalid;

2.5.3 RFM_SET_GET_EAS_MASK ( Set /Get Gate Channel EAS Specific Matching Data )

Set/get gate channel specific match data: start address, length and data.

- Format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR​ | CMD | LEN | OPTION | EASADDR | EASLEN | EASDATA |  |
| 0xCF | 0xFF | 0x00 8 4 | 1or35 | 1 Byte | 1 Byte | 1 Byte | N Bytes​ | 2 Bytes​ |

Parameter explanation:

- Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

- EASADDR: The starting address of EAS specific matching data, 1 byte in length, with a value of [0,31].

- EASLEN: The length of EAS specific matching data, 1 byte, value [0,32].

- EASDATA: EAS specific matching data, 32 bytes in length. The first EASLEN bytes are EAS specific matching data, and the remaining bytes are filled with 0x00.

- Response format and status byte

Set the response:

| HEAD | ADDR | CMD | LEN | STATUS | OPTION | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0084 | 0x02 | 1Byte | 1Byte | 2Byte |

获取响应：

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION | EASADDR | EASLEN | EASDATA |  |
| 0xCF | 0x00 | 0x00 8 3 | 36 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | N Bytes​ | 2 Bytes​ |

  - STATUS:

- 0x00: command executed successfully;

- Other values: invalid;

Appendix A. Operational Status Returned by Tags

code returned by the tag is 8 bits in total, see Table A-1.

Table A-1 Operation status returned by the tag

| Operation status code | Operational Status | describe | Error Priority |
| --- | --- | --- | --- |
| 0x81 | Other Errors | Unknown error returned by tag |  |
| 0x82 | Storage area overflow | The target storage area does not exist |  |
| 0x83 | Storage Area Lock | Write operation to the memory area that is locked as non-writable Perform a read or erase operation on a storage area that is locked as unreadable. |  |
| 0x84 | Insufficient power | The tag does not have enough energy to complete the operation |  |
| 0x85 | Non- specific error | Unknown error returned by tag |  |

Appendix B. CRC16 checksum reference C code

as follows:

#define PRESET_VALUE 0xFFFF

#define POLYNOMIAL 0x8408

unsigned int uiCrc16 Cal( unsigned char const * pucY , unsigned char ucX )

{

unsigned char ucI,ucJ ;

unsigned short int uiCrcValue = PRESET_VALUE;

for( ucI = 0; ucI < ucX ; ucI ++)

,

uiCrcValue = uiCrcValue ^ *( pucY + ucI );

for( ucJ = 0; ucJ < 8; ucJ ++)

,

if( uiCrcValue & 0x0001)

,

uiCrcValue = ( uiCrcValue >> 1) ^ POLYNOMIAL;

}

else

{

uiCrcValue = ( uiCrcValue >> 1 );

}

}

}

return uiCrcValue ;

}

Appendix C. Definition of STATUS.

Table A-6 STATUS definition

| STATUS | Error description |
| --- | --- |
| 0x00 | Execution successful (this only means that the module successfully received the tag response data. If there is a tag execution status in the tag response, it should be further determined whether the tag execution status is correct) |
| 0x01 | The parameter value is incorrect or out of range, or the module does not support the parameter value. |
| 0x02 | Command execution failed due to an internal module error |
| 0x03 | reserve |
| 0x12 | No inventory was performed to the tag or the entire inventory command was completed. |
| 0x14 | Tag response timeout |
| 0x15 | Demodulation tag response error |
| 0x1 6 | Protocol authentication failed |
| 0x1 7 | Wrong password |
| 0xFF | No more data |